Accelerating Graph Algorithms with Priority Queue Processor

Ch’ng Heng Sun1, Chew Eik Wee2, Nasir Shaikh-Husin3, Mohamed Khalil Hani4

VLSI-ECAD Research Laboratory (P04-Level 1), Microelectronic and Computer Engineering Department (MiCE), Faculty of Electrical Engineering (FKE),

Universiti Teknologi Malaysia.

1chnghengsun@yahoo.com.sg

2eikweechew@hotmail.com

3nasirsh@utm.my

4khalil@fke.utm.my

Abstract

Graphs are a pervasive data structure in computer science, and algorithms working with them are fundamental to the field. Of the various graph algorithms, techniques for searching a graph are  the heart of graph algorithms. Many graph algorithms are organized as simple elaborations of basic graph searching algorithms. For the searching of a graph, Priority Queue is used to maintain the tentative search result and choice of priority queue implementation would significantly affect the run-time and memory consumption of a graph algorithm. In this work, we demonstrate how to accelerate graph algorithms using priority queue processor. Dijkstra’s algorithm is chosen as the target implementation, as many state-of-the-art graph algorithms use Dijkstra’s algorithm at the heart of their computational engine. Assuming embedded hardware-software co- design, results  show that  our  priority  queue processor  performs  better  than  software  implementation, and  the run-time complexity of Dijkstra’s algorithm is reduced from O(n lg n) in software implementation to O(n) with our priority queue processor.

Keywords:

Graph Algorithms, Shortest Path Routing, Priority Queue, Priority Queue Processor

1.   Introduction

Graphs are a pervasive data structure in computer science, and algorithms working with them are fundamental to the field, among others; Depth-First Search, Breadth- First Search, Topological Search, Spanning Tree Algorithms, Dijkstra’s Algorithms, Bellman-Ford Algorithms, Floyd-Warshall Algorithm, etc. In real world applications, there exist many algorithms which are actually extended from these basic graph algorithms. For instance, in the field of VLSI physical design automation, there are many interesting computational problems defined in terms of graph; among others, Lee’s Algorithm, Maze Routing Algorithms, Matching Algorithms, Min-Cut and Max-Cut Algorithms, Minimum Steiner Tree Algorithms, Span Minimum Tree Algorithms, Clock Skew Scheduling, Clock Net Synthesis, Critical Net Routing, etc.

A graph, G = (V, E) consists of |V| numbers of vertices/nodes and |E| numbers of edges. Real world problems modeled in mathematical set can be represented as graphs, where elements in the set are represented by vertices, and the relation between any two elements are represented   by   edges.   The   run-time   complexity   and

memory-consumption of graph algorithms are expressed in terms of |V| and |E|. A graph searching algorithm can discover much about the structure of a graph and many graph algorithms are organized as simple elaborations of basic graph searching algorithms [1]. Searching a graph means systematically following the edges of the graph so as to visit the vertices of graph. For the searching of a graph, Priority  Queue  is used to maintain  the tentative search result and choice of priority queue implementation would significantly affect the run-time and memory consumption of a graph algorithm [2].

Priority Queue is an abstract data structure to maintain a set of elements, where all elements are arranged in accordance to their associate-priority. The associate-priority can  be  given as  time-of-occurrence, level-of-importance, physical-parameters, delay/latency, etc, depending on target application. Two basic operations are supported by priority queue, namely (i) INSERT(Q, x), which is generally referred  to  as  ENQUEUE  operation,  and  (ii) EXTRACT(Q) which is sometimes referred to as DEQUEUE  operation. The performance of priority queue operations are measured in terms of n, where n refers to the total number of elements in the queue.

In  many advanced algorithms where items/tasks are processed according to a particular order, priority queue has proven to be very useful. For task-scheduling  on a multi- thread, shared-memory computer; priority queue is used to schedule   and   keep   track   of   the   prioritized   pending processor tasks/threads. In the case of discrete-event- simulation (DES), priority queue is used where items in the queue are pending-event-sets, each with associated time-of- occurrence that serves as priority; many simulators, emulators, and  synthesizers are designed based on  DES concept. In the case of shortest-path based graph problem, priority queue has been used extensively in QoS internet packet routing, weighted shortest path problem, multi- constrained routing, VLSI routing, PCB routing, etc.

In this paper, choices of priority queue implementation will be discussed in section 2. Section 3 introduces the priority queue processor in this work to accelerate the graph algorithm. Section 4 illustrates the Dijkstra’s Algorithm. The embedded system architecture featuring the priority queue  processor  is  described  in  section  5.  Section  6 presents the result of this work.

2.   Choices of Priority Queue

As shown in Figure 1, operation INSERT(Q, x) inserts a new element, x into priority queue, Q; meanwhile EXTRACT(Q) returns the element with the highest priority. Preferably,  in   some  circumstances  where  the  highest priority   is   given   by   a   minimum   value,   the   term EXTRACT-MIN(Q)  is used instead of the more general term EXTRACT(Q); whereas in other cases where the highest   priority   is   the   maximum   value,   the   term EXTRACT-MAX(Q)  is used. Hereinafter, EXTRACT(Q) is used interchangeably with EXTRACT-MIN(Q)  or EXTRACT-MAX(Q).

INSERT (Q, x)

Î Insert new element, x into queue Q.

EXTRACT (Q)

Î Remove and return the highest priority element in queue Q.

MAX(Q),   then  simply  solve  it   by  inverting  all   the associate-priority (1/priority) before insertion into queue.

The simplest way to implement a priority is to keep an associate array mapping each priority to a list of items with that priority, see Figure 2. The priorities are held in static array which stores the pointers to the list of items assigned with that priority. Such implementation is static, for example,   if   the   allowed   priority   ranged   from   1   to

4,294,967,295 (32-bit) then an array of (4 Giga-length) * (size of pointer storage, i.e. 32-bit) is consumed, a total of

16 Gigabytes is gone, just to construct a priority data structure.

Priority               List of Elements

A

D          B           E          C NIL

Z          G NIL

H           J           V          Z K

NIL 1

2

3

4

5

6

7

8

Figure 2. Simplest way to implement Priority Queue

A more flexible and practical way to implement a priority queue is to use dynamic array/queue, see Figure

3(a), which means the length of array does not depend on

the range of priority. Each INSERT(Q,  x) will extend the existing queue-length by one unit (n Å n + 1), put in the item with priority x, then sort the whole queue to ensure the highest priority item is ready if EXTRACT(Q)  is invoked. Such sorting during insertion will take up to O(n) computation  run  time.  For  extraction,  since  the  whole queue already sorted during insertion, extraction takes constant O(1) time.

Figure 1. Basic Priority Queue Operations

For works done to implement a priority queue, either one of EXTRACT-MIN(Q)  or EXTRACT-MAX(Q) function will be implemented, depending on what application is targeted. While software implementation could  be  easily  modified  to  switch  from  EXTRACT-

index, i

1      2       3       4       5       6       7

| 2 | 3 | 8 | 12 | 16 | 25 | 38 |
| --- | --- | --- | --- | --- | --- | --- |

Priority

MIN(Q) to EXTRACT-MAX(Q),  or the other way round, it is not the case for full-custom hardware implementation. Anyway, this is not a big issue since maximum can be

treated  as   reciprocal  to   the   minimum,  or   vice-versa

(maximum = 1/minimum). For instance, given a priority queue which only provides INSERT(Q)  and EXTRACT- MIN(Q),  but  the  target-application  needs  EXTRACT-

Figure 3(a). Priority Queue, implemented as array.

If a self-balanced Binary-Heap is used, see Figure 3(b), each INSERT(Q,  x) or EXTRACT(Q)  take O(lg n) run- time. In order to achieve better priority queue performance, many novel approaches have been taken to implement different structure of heap, i.e. Binomial-Heap, Fibonacci- Heap,  Relaxed-Heap,  Parallel-Heap,  to  name  a  few.  A

developer has a number of choices with different access speed, memory consumption, and required hardware platform. Theoretically, Fibonacci-Heap Priority Queue (“FHPQ”) has better performance compared to other types of heap, especially for applications where the number of EXTRACT and DELETE operations are relatively small compared to INSERT and DECREASE-KEY operations; however, the drawback of Fibonacci-Heap is its complexity in practice with larger constant-time-factor.

achieved better clocking rate. Having said that, all these designs are implemented on hardware high-speed network routers, they are different from embedded platform requirement; such as in our context, graph algorithm computation (or any other DES). For example, all these designs are targeting small number of priorities (i.e. 8-bit), but in the case for graph algorithm; the data which represents the priority can be in any range (i.e. 32-bit); furthermore, for embedded system implementation, those device are practically not accessible to us.

Root

1    2

2    3                        3    8

4                5               6                7

3.   Systolic Array Priority Queue Processor

In a programming model, priority queue can be viewed as an array; when new element is inserted, it will be compared with all  other elements in  queue, in  order to determine where to slip in this new element, in such a way that priority-order of the queue is always maintained. This approach is known as insertion-sort priority queue. Derived from   the   insertion-sort  algorithm,   O(n)   run-time   per

12           16

25           38

operation is needed for a priority queue with n elements.

Figure 3(b). Priority Queue, implemented as heap. (in this case, Binary-Heap)

Due to the tremendous application of priority queue, there are a lot of researches done on how to accelerate the priority queue operation using custom hardware resources. There are two main categories of this work; (a) proposed full-custom parallel hardware design to accelerate array- based priority queue, (b) no specific hardware designed, but assuming PRAM (Parallel Random Access Machine) model, and propose new heap data structure executing on these PRAM. The former often achieves very high throughput and clocking frequency, favorite choice to high speed applications such as QoS network routing and real-time applications; whereas the latter often ignore the severe memory communication overhead and gives theoretical improvement in terms of run-time complexity.

Due to limited space, works in (b) is less related to our design work, it will be excluded, reader could refer to [3] for details. On the other hand, works in (a) includes Binary Trees of Comparator by [4]; such hardware priority queue is not efficient if the queue size is large, large fan-in for Binary-Tree also caused severe bus loading effect when it comes  to  physical implementation. Another approach to implement priority queue is using FIFO  Priority Queue as proposed by [5] and [6], but the main drawback is  the priority range is not flexible, just like the illustration in figure 2, thus not practical if the allowed priority range is large. Shift Register implementation of priority queue [6], [7] has better performance compared to Binary Trees of Comparators and FIFO Priority because the priority level and the queue size can be easily scaled, but Shift Register also suffers severe bus loading effect when it comes to real hardware  implementation. To  overcome  the  bus-loading effect of Shift Register, A hybrid design of Systolic-Shift- Register priority queue processor had been proposed by [8], which claims to reduce bus loading effect and could have

Given an all-pair-single-source shortest-path graph problem to be solved with the Dijkstra’s algorithm, the run- time complexity is O(n2)  if insertion-sort priority queue (hereinafter called “ISPQ”) is used; the run-time can be improved to O(n lg n) if the priority queue is implemented as binary-heap priority queue (“BHPQ”). This shows that different choices of priority queue implementation could significantly affect the overall performance of graph algorithm, especially when the queue size is very large.

An in-house-developed hardware, Systolic Array Priority Queue Processor (“SAPQ”) could further reduce the run-time complexity of the said graph algorithm to O(n). The SAPQ, benefited from parallelizing a modified version of insertion sort algorithm supports both INSERT(Q,  x) and EXTRACT(Q)  in constant O(1) run time. Improved from the Systolic-Shift-Register by [8], the architecture of SAPQ is much simpler, localizing control-unit and data- path for each processing element (PE) making it highly pipelined, easy for cascading, and with no bus-loading effect.

The SAPQ consists of an array of identical processing elements (PEs), with each PE holding a single queue- element. The identical-PE feature of SAPQ makes it very flexible, where a parameterized design always allows generation of any queue-size in a FPGA environment. A very  large  queue-size  can  be  achieved  by  cascading multiple FPGAs on a single board or distributed through multiple boards. The SAPQ employs n number of PEs for worst-case n-size priority queue. Each PE is interconnected to only its immediate-neighbours. Figure 4 illustrates the top-level architecture of SAPQ. Of all the PEs, only the left-most  PE  will  communicate with  the  outside  world, meaning the new element is inserted to PE1, as well as the highest priority element will be extracted from PE1.

INSERT

EXTRACT

Systolic Array Priority Queue Processor

…..

DIJKSTRA(G, w, s){

for (each vertex v ɽ V[G] and v  s){

d[v] Å 

ʌ[v] Å NIL

}

-MIN                   PE1           PE2           PE3                    PEn

Figure 4. Systolic Array Priority Queue Processor

Each INSERT and EXTRACT-MIN operation on SAPQ completes in 3 clock cycles. The design of SAPQ allows, for each queue element, the element-ID in 32-bit, so as the associate-priority in 32-bit. The element-ID can be treated  as  associate-pointer which  points  to  a  block  of satellite-data of that element.

The   SAPQ   design   is   compiled   and   targeted   to ALTERA Stratix and Cyclone II FPGA devices. The synthesis shows that the design can obtain 240 MHz clocking frequency in Stratix and 175 MHz in Cyclone II. The design achieved 5.12 Gbps throughput rate for Stratix and 3.73 Gbps in Cyclone II. For details of SAPQ design, refer to [9].

4.   Dijkstra’s Graph Algorithm

All shortest path based graph algorithms follow a sequence of predefined graph-search procedures to ensure the shortest paths are found (if any) within the minimum period  of  time.  In  this  paper,  Dijkstra’s  algorithm  is selected for illustration purpose. Refer the pseudo-code in Listing 1.

Given a graph G = [V, E], where V[G] denotes the set of vertices and W[G] denotes the set of edge-weights. We use s to denote the source-vertex, for any two adjacently- connected vertices, v = Adj[u] or u = Adj[v]. d[u] denotes total-path-length from s to u, whereas d[v] denotes total- path-length from s to v. Given w(u, v) denoted the edge- weights from u to v, then d[v] = d[u] + w(u, v). Specifically for shortest path algorithm, S is the set used to hold the nearest vertex from source, ʌ[v] is used to hold the precedent-vertex of v. Upon complete execution of algorithm, the shortest path from s to v can be traced by dereference ʌ[v] backward to source, and the total-path- length is given by d[v].

Dijkstra’s algorithm begins with source-node, where all nodes adjacent to source will be scanned and the edge- weights inserted into priority queue. Then the edge with minimum-weight will be extracted from priority queue, and the node in which this minimum-weight-edge heading-to becomes active. The same scanning mechanism continues at this current active node, with insertion to priority queue if there is no identical entry in queue, or relaxation (decrease-key) if identical entry already exists in queue. The procedure continues until all nodes have been visited.

d[s] Å 0

INSERT(Q, s, d[s])

S Å Ø

do{

(u, d[u]) Å EXTRACT-MIN(Q)

S Å S U {u}

for (each vertex v ɽ Adj[u]){

if (d[v] = ){

d[v] Å d[u] + w(u, v)

ʌ[v] Å u

INSERT(Q, v, d[v])

}

elseif (d[v] > d[u] + w(u, v)){

d[v] Å d[u] + w(u, v)

ʌ[v] Å u

DECREASE-KEY(Q,  v, d[v])

}

}

}(while Q  Ø)

}

Listing 1. Pseudo-code of Dijkstra’s Algorithm

DECREASE-KEY (Q, x, new(x_priority))

Î Find element x in queue, Q;

If (new(x_priority) dominates old(x_priority))

x_priority Í new(x_priority).

Figure 5. Decrease-Key function

Notice that besides the basic priority queue operations in Figure 1, the graph algorithm needs one additional function, the DECREASE-KEY. The DECREASE-KEY function is very important because it is necessarily for any graph  algorithm  to  perform  RELAXATION   (see  [1]). Recall each element x in queue Q comes with its ID (here, denoted as x_ID) and its associate-priority (here, denoted as x_priority). Refer to Figure 5, when it comes to a condition where  the  associate-priority, x_priority  of  an  element x needs update, the DECREASE-KEY is invoked to find that element x in queue; if the new-associate-priority-of-x, new(x_priority) possesses higher priority than the old- associate-priority-of-x, old(x_priority) (the one exist in queue), then replace that associate-priority-of-x. Such property  is  called  dominancy,   and  it  is  the  key  to understand graph algorithms.

DIJKSTRA_MODIFIED(G, w, s){

for (each vertex v ɽ V[G]){

d[v] Å 

ʌ[v] Å NIL

}

d[s] Å 0

INSERT(Q, s, d[s])

S Å Ø

do{

do{

(u, temp) Å EXTRACT-MIN(Q)

}(while d[u]  temp)

S Å S U {u}

for (each vertex v ɽ Adj[u]){

if (d[v] = ){

d[v] Å d[u] + w(u, v)

ʌ[v] Å u

INSERT(Q, v, d[v])

}

elseif (d[v] > d[u] + w(u, v)){

d[v] Å d[u] + w(u, v)

ʌ[v] Å u

INSERT(Q, v, d[v])

}

}

}( while Q not empty)

}

Listing 2. Modified version of Dijkstra’s Algorithm

which is no longer valid. (ii) During EXTRACT-MIN, the returned queue-entry will be checked for validity, until a valid entry is returned. Such modification retains the relaxation property of graph algorithm, except the priority queue size at some instance might grow larger than algorithm in Listing 1. Anyway, such exception does not have profound effect on SAPQ since the operation run-time is constant, unlike ISPQ, BHPQ, or other heaps which the operation run-time depends on queue-size.

5.   Embedded System Implementation

Targeted for embedded system implementation, SAPQ will serve as co-processor, to off-load the recursive priority queue access and maintenance from the host processor. Our implementation, assuming a general purpose processor, NIOS, to serve as host processor, executing (modified) Dijkstra’s algorithm, with the priority queue access and maintenance fully handled by the SAPQ. The whole system is prototyped on a FPGA as illustrated in Figure 6. The Avalon Interface Unit is designed to handle the communication protocol between host processor and SAPQ. The  embedded  system  could  be  deployed  to  handle  all kinds of application which utilize priority queue. The Dijkstra’s algorithm executed by the host processor can be replaced by other algorithms such as discrete-event simulation,  global-positioning-system, mobile  navigating, etc.

Due to the limitation of Avalon bus bandwidth, which is 32-bit width, the SAPQ does not actually execute at its

64-bit interfacing capability. An optimal SAPQ could be achieved   by   having   it   implemented   in   higher-end

communication bridge, such as PCI-64 as the system bus.

Priority Queue Co-Processor

Host

Processor

All software implementation of priority queue in heap (Binary-Heap, Binomial-Heap, and Fibonacci-Heap) could easily incorporate all three functions needed in finding the shortest  path:  INSERT,  EXTRACT-MIN,  and DECREASE-KEY. However, neither hardware implementation    of     priority    queue    ever     provides

NIOS

Avalon Interface Unit

Systolic

Array Priority Queue Processor

DECREASE-KEY function; this function is possible to be implemented but often ignored due to additional hardware resources required. The design of SAPQ also does not incorporate DECREASE-KEY function. Hence, modification on the targeted Dijkstra’s graph algorithm is needed, so that the algorithm does not invoke DECREASE- KEY directly for relaxation, but indirectly perform relaxation using the only available INSERT(Q, x) and EXTRACT-MIN(Q).

The modified version of Dijkstra’s algorithm is presented in Listing 2, which only invoked INSERT and EXTRACT-MIN functions. The sequence of execution remains  except  (i)  when  supposed DECREASE-KEY is needed, INSERT is invoked instead; this causes the queue size to actually grow by one, and there is one entry in queue

Avalon System Bus

Figure 6. Embedded System on FPGA

6.   Result

SAPQ with queue-size of 200 entries is implemented on the said embedded system; this queue-size is constrained by  the  logic resources of  this  particular FPGA. Several software priority queues (ISPQ, BHPQ, and FHPQ) are used to compare the performance of SAPQ. Having the SAPQ actually running at lower than optimal speed (50

MHz compared to the maximum allowed is 240 MHz), narrower bus bandwidth (32-bit compared to allowed 64- bit), and high redundancy in cycles per operation incurred

by the host processor (44 cycles for INSERT and 52 cycles

for EXTRACT; compared to actually 3 cycles designed for

INSERT and 3 cycles designed for EXTRACT), the gain achieved in terms of number of cycles spent per operation is still significant.

| Table 1: Comparisons in terms of run-time complexity. (Queue Size = n) | Resource Utility |  | Run Time Complexity, n |  |
| --- | --- | --- | --- | --- |
|  | # of Processor | # of Memory Storage | INSERT | EXT- MIN |
| Systolic Array Priority Queue, SAPQ | O(n) | NIL | O(1) | O(1) |
| Binary-Heap Priority Queue, BHPQ | O(1) | O(n) | O(lg n) | O(lg n) |
| Insertion-Sort Priority Queue, ISPQ | O(1) | O(n) | O(n) | O(1) |
| Fibonacci-Heap Priority Queue, FHPQ | O(1) | O(n) | O(1) | ∗ O(lg n) |

| Table 2: Comparison in terms of processor cycles. Refer to Table 1, the performance of priority queue (Queue size limited to n=200) | WORST CASE (number of cycles) |  | SPEED UP GAIN (Achieved by SAPQ) |  |
| --- | --- | --- | --- | --- |
|  | INSERT | EXT- MIN | INSERT | EXT- MIN |
| Systolic Array Priority Queue, SAPQ | 44 | 52 | 1.00 | 1.00 |
| Binary-Heap Priority Queue, BHPQ | 753 | 1185 | 17.11 | 22.79 |
| Insertion-Sort Priority Queue, ISPQ | 13674 | 65 | 310.77 | 1.25 |
| Fibonacci-Heap Priority Queue, FHPQ | 799 | 44262 | 18.16 | 851.19 |

depends on the queue-size, n. In Table 2, the worst case INSERT is for inserting entries so that the queue is full. The worst case EXT-MIN is for extracting the minimum entry when the queue is full.

Compared to BHPQ, the speed up gain achieved by SAPQ is more than expected. Theoretically, one could expect lg n speed up (which in this case n = 200, log2 200 =

7.6) by SAPQ; this real implementation achieved 17 times

and 22 times worst-case speed up for INSERT and EXTRACT operation. This is because software implementation  suffers  severe  memory  communication

overhead, where tremendous cycles are spent to access heap

data structure stored in memory; whereas SAPQ, having all the elements stored in registers, such communication drawback is eliminated.

For FHPQ, the O(lg n) for EXT-MIN is worst-case amortized-time, refer [1] for amortized analysis.

∗

Next, compared to ISPQ, SAPQ obtained 310 speed gains for worst-case INSERT and 1.25 gains for EXTRACT. Notice the 310 times speed gain is also greater than theoretical expectation; it is also due to the advantage of hardware implementation which eliminates the memory communication overhead.

Lastly, comparison is made between the theoretically most efficient priority queue, the FHPQ with our SAPQ; the result is very impressive. Both FHPQ and SAPQ claims O(1) run-time complexity for INSERT, this implementation shows 18 times gain achieved using SAPQ because constantly large  cycles are spent by  FHPQ to  handle a bunch of pointer manipulation. Similarly, for EXTRACT operation, the speed-up gain achieved by SAPQ is over 851 times.

The above reported worst case speed-up gain could be more if larger SAPQ were to be implemented (i.e. queue- size  of  1000).  The  main  drawback  of  Systolic  Array Priority Queue Processor is the logic resources consumed, when other software implementation of software priority queues only take space in random access memory. Anyway, for real-time applications where the speed is top-priority, the drawback in logic consumption is a worthy trade-off.

References

[1]   T. H. Cormen, C. E. Leiserson, R. L. Rivest, Clifford Stein, Introduction To  Algorithms, 2nd   Edition,  The  MIT  Press, McGraw-Hill Book Company, 2001.

[2]   S.  S.  Skiena,  The  Algorithm  Design  Manual,  Springer- Verlag, New York, 1997

[3]   Ch’ng Heng Sun, Mohamed Khalil Hani, “Design of a Graph Processor for VLSI Routing”, Master Research Proposal, FEB. 2006.

[4]   D. Picker and R. Fellman, “A VLSI Priority Packet Queue with Inheritance and Overwrite,” IEEE Trans. Very Large Scale Integration Systems, vol. 3, no. 2, pp. 245-252, June

1995.

[5]   R. Brown, “Calendar Queues: A Fast O(1) Priority Queue Implementation for the Simulation Event Set Problem,” Comm. ACM, vol. 31, no. 10, pp. 1220-1227, Oct 1988.

[6]   J. Chao, “A Novel Architecture for Queue Management in the ATM Network,” IEEE J. Selected Areas in Comm., vol. 9, no. 7, pp. 1110-1118, Sept. 1991.

[7]   K.  Toda,  K.  Nishida,  E.  Takahashi,  N.  Michell,  and  Y.

Tamaguchi, “Design and Implementation of a Priority Forwarding Router Chip for Real-Time Interconnect Networks,” Int’l J. Mini and Microcomputers, vol. 17, no. 1, pp. 42-51, 1995.

[8]   S.W.  Moon,  J.  Rexford,  K.G.  Shin,  “Scalable  Hardware Priority Queue Architectures for High-Speed Packet Swicthes”, IEEE Trans. On Computers, vol. 49, no. 11, Nov.

2000.

[9]   Ch’ng Heng Sun, Mohamed Khalil Hani, “Systolic Array

Priority Queue Processor”, Research Report, APR. 2006.